Project Report: Network Simulator

COMPUTER NETWORKS LAB — ITL 351

| Semester | Spring 2026 |
| --- | --- |
| Date | 25-03-2026 |
| Submitted To | Dr. Iqra Altaf Gillani |
| Submitted By | Anshuman Singh Raghuvanshi, Prakhar Pandey |
| Enrollments | 2023BITE038, 2023BITE080 |
| Language | C++17 |

# Objective

The aim of this project is to create a console-based network simulator in C++ that imitates how devices communicate in a network. The specific objectives are:

- Implement Physical Layer components: end devices, hubs, and direct connections

- Implement Data Link Layer components: bridge and switch with MAC address learning

- Simulate hub flooding and switch selective forwarding

- Implement CRC-16 error detection for frame-level error control

- Implement CSMA/CD as the medium access control protocol

- Implement Go-Back-N sliding window protocol for flow control

- Report broadcast and collision domains for different topologies

# Introduction

This project involves developing a basic network simulator that demonstrates the working of the Physical Layer and Data Link Layer of the OSI model. The simulator is implemented entirely in C++17 as a console-based application and creates virtual network devices such as end devices, hubs, bridges, and switches.

The Physical Layer simulation handles bit-level representation of data (ASCII to binary), creation of devices and connections, and topology formation. The Data Link Layer simulation handles framing, MAC address learning, error detection using CRC-16, medium access control using CSMA/CD, and flow control using the Go-Back-N sliding window protocol.

# Tools Used

| Tool / Concept | Details |
| --- | --- |
| Programming Language | C++17 |
| Compiler | g++ / clang++ (Apple LLVM) |
| IDE / Editor | Visual Studio Code |
| Physical Layer | End devices, Hub, direct link connections |
| Data Link Layer | Switch, Bridge, MAC learning, framing |
| Error Control | CRC-16 (CCITT) — cyclic redundancy check |
| Access Control | CSMA/CD with exponential backoff |
| Flow Control | Go-Back-N sliding window protocol |
| Topology | Star, direct link, dual-star via switch |

# System Design / Methodology

The project is implemented in C++17 using an object-oriented approach. The class hierarchy and key components are described below.

## Class Architecture

| Class | Responsibility |
| --- | --- |
| NetworkDevice (base) | Abstract base class for all network devices. Holds name, type, and virtual receiveFrame() method. |
| EndDevice | Represents a host with a unique MAC address. Can send raw bits (Physical Layer) and DLL frames with CRC + CSMA/CD + Go-Back-N. |
| Hub | Physical layer repeater. Floods incoming frames to all ports except the source port. |
| Bridge | Data Link layer device with 2 ports. Learns MAC addresses and selectively forwards or filters frames. |
| Switch | Multi-port Data Link layer device. Maintains a MAC address table and forwards frames only to the correct port. Floods on unknown destination. |
| Network | Global topology manager. Stores all devices and connections, computes broadcast/collision domains. |
| Frame | Data Link PDU containing src MAC, dst MAC, payload, CRC-16 checksum, sequence number, and ACK fields. |
| Connection | Represents a physical link between two devices with a busy/idle state for CSMA/CD. |

## Physical Layer

The Physical Layer is simulated through the EndDevice::sendRaw() method. When a device sends data, the payload is converted to an 8-bit binary representation (ASCII to binary string) and transmitted over the connected link. Hubs receive frames and flood them to all connected ports except the source, exactly mirroring the real-world hub behaviour.

## Data Link Layer

The Data Link Layer is implemented through three protocols working together:

1. Error Control — CRC-16 (CCITT)

Every frame carries a 16-bit CRC checksum computed over its payload using the CRC-16/CCITT polynomial (0x1021). On receipt, the receiver recomputes the CRC and compares it with the transmitted value. A mismatch results in the frame being dropped and an error message logged.

2. Access Control — CSMA/CD

Before transmitting a frame, the sender checks whether the channel is idle. If the channel is busy (another frame is in transit on the same link), the sender waits using a simulated exponential backoff and retries up to 5 times. If the channel is idle, transmission proceeds and the link is marked busy until the frame is delivered.

3. Flow Control — Go-Back-N Sliding Window

The sender transmits up to W frames (configurable window size) without waiting for individual acknowledgements. The receiver sends a cumulative ACK for each correctly received frame. If a frame is lost or corrupted, all frames from that sequence number onwards are retransmitted. The simulator demonstrates window-based pipelining with sequence numbers printed for each frame.

## Broadcast and Collision Domains

The Network class computes broadcast and collision domains automatically for any topology:

- Hub: all connected ports share one broadcast domain and one collision domain.

- Switch/Bridge: each port is a separate broadcast domain and separate collision domain.

- Direct link: one collision domain per link.

# Test Cases

## TC-1: Two End Devices — Direct Link (Physical Layer)

Topology: DevA ←——→ DevB

Purpose: Verify that two end devices can exchange data over a direct physical link with correct bit representation.

Expected result: Both devices receive each other's messages. Bit string displayed for each transmission.

| Metric | Expected | Result |
| --- | --- | --- |
| Broadcast Domains | 2 | 2 |
| Collision Domains | 1 | 1 |
| Data received by DevB | Yes | Yes |
| Bit representation shown | Yes | Yes |

## TC-2: Star Topology — 5 End Devices + Hub (Physical Layer)

Topology: PC1–PC5 all connected to Hub1.

Purpose: Verify hub flooding behaviour — when one device transmits, all other devices receive the frame.

Expected result: PC1's broadcast reaches PC2, PC3, PC4, PC5. Hub does not filter.

| Metric | Expected | Result |
| --- | --- | --- |
| Broadcast Domains | 1 | 1 |
| Collision Domains | 1 | 1 |
| Devices receiving frame | 4 (all except sender) | 4 |
| Hub floods all ports | Yes | Yes |

## TC-3: Switch + 5 End Devices — DLL Protocols

Topology: Host1–Host5 all connected to SW1.

Purpose: Demonstrate MAC address learning, CRC-16 error detection, CSMA/CD access control, and Go-Back-N sliding window (window=4). Host1 sends 6 frames to Host5; Host3 then sends to Host2.

| Metric | Expected | Result |
| --- | --- | --- |
| Broadcast Domains | 5 | 5 |
| Collision Domains | 5 | 5 |
| MAC learning (SW1) | Learns src on each frame | Yes |
| CRC on each frame | Checked | Yes — CRC-16 verified |
| CSMA/CD | Channel idle before TX | Yes — logged per frame |
| Go-Back-N window=4 | 4 frames pipelined | Yes — ACKs after window |
| All 6 frames delivered | Yes | Yes |

## TC-4: Two Hub-Stars Connected via Switch (10 Devices)

Topology: A1–A5 → HubA → SW_Core ← HubB ← B1–B5

Purpose: Demonstrate inter-segment communication and correct domain counts when two hub-based star topologies are connected through a switch.

| Metric | Expected | Result |
| --- | --- | --- |
| Broadcast Domains | 2 | 2 |
| Collision Domains | 2 | 2 |
| A1 reaches B3 | Yes (via HubA→SW→HubB) | Yes |
| Switch MAC learning | Learns HubA-side MAC | Yes |

## TC-5: CRC Error Detection Demo

Purpose: Verify that a deliberately corrupted frame is detected and dropped by the receiver.

| Frame | Expected | Result |
| --- | --- | --- |
| Good frame | CRC PASS — delivered | PASS ✓ |
| Corrupted frame | CRC FAIL — dropped | FAIL ✗ — dropped |

## TC-6: Go-Back-N Sliding Window Demo

Purpose: Demonstrate Go-Back-N with window size 3 over 8 packets. Sender pipelines up to 3 frames before waiting for ACKs.

| Metric | Expected | Result |
| --- | --- | --- |
| Window size | 3 | 3 |
| Total packets | 8 | 8 |
| All delivered | Yes | Yes |
| Seq numbers logged | 0–7 | 0–7 |

# Result

All six test cases executed successfully. The simulator correctly demonstrates:

- Physical layer bit transmission and hub flooding behaviour

- Switch MAC address learning and selective forwarding

- CRC-16 error detection — corrupted frames are identified and dropped

- CSMA/CD — channel is sensed before each transmission with exponential backoff on collision

- Go-Back-N — configurable window size with pipelined frame delivery and cumulative ACKs

- Correct broadcast domain and collision domain counts for all topologies

# Conclusion

This project successfully implements a console-based network simulator in C++17 that covers both the Physical Layer and the Data Link Layer of the OSI model. The simulator demonstrates hub and switch behaviour, MAC address learning, CRC-16 based error detection, CSMA/CD access control, and Go-Back-N sliding window flow control.

Through the six test cases, we verified that the simulator behaves correctly across different topologies including direct links, hub-based star networks, switch-based networks, and interconnected dual-star topologies. The project deepened our understanding of how data travels across a network and how devices at different layers manage communication efficiently.

# References

- Forouzan, B. A. (2013). Data Communications and Networking (5th ed.). McGraw-Hill.

- Tanenbaum, A. S. (2010). Computer Networks (5th ed.). Prentice Hall.

- Stroustrup, B. (2013). The C++ Programming Language (4th ed.). Addison-Wesley.

- IBM Documentation. https://www.ibm.com/docs

- Artificial Intelligence — used for debugging and formatting assistance.